	[image: image7.png]Vy PATROLLINE 4





	FITTING 

INSTRUCTION
	Rev. 1

	
	
	

	Can Bus Diagram:  

Audi A3 2012(2020
	[image: image1.jpg]




	DIP SWITCHES SELECTION AND SOFTWARE
	

	from CARLIST29
and next

	
	[image: image2.jpg]


7 [image: image3.jpg]


9
	

	
	
	
	

	
	


ELECTRIC WIRING BETWEEN ALARM AND VEHICLE

	Power

See picture 1
	Red Wire:
	Connect to the RED/PURPLE wire pos.73 of the “A” 73 pins BLACK connector, linked to the BLACK central unit, fixed under the dashboard at the left, on the driver side.

	
	Black Wire:
	Connect to the BROWN wire pos. 63 of the “A” 73 pins BLACK connector, linked to the BLACK central unit, fixed under the dashboard at the left, on the driver side.

	Enabling

CAN BUS

See picture 1
	White/Brown Wire:
	Connect to the ORANGE/BROWN wire pos.17 of the “A” 73 pins BLACK connector, linked to the BLACK central unit, fixed under the dashboard at the left, on the driver side. The wire is braided with the ORANGE/GREEN wire.

	
	White/Green Wire:
	Connect to the ORANGE/GREEN wire pos.16 of the “A” 73 pins BLACK connector, linked to the BLACK central unit, fixed under the dashboard at the left, on the driver side. The wire is braided with the ORANGE/BROWN wire.

	Hazard

See picture 2
	Yellow/Green Wire:
	Connect to the PURPLE/RED wire pos.42 of the “C” 73 pins connector, linked to the BLACK central unit, fixed under the dashboard at the left, on the driver side.

	Horn Command

See picture 3
	White/Red Wire:
	Attention NEGATIVE command: please select properly the polarity output of the siren before to connect it.
Connect to the BROWN/RED wire pos. 24, of the 48 pins BLACK connector marked “B”, linked to the BCM central unit, under the dashboard on the driver’s side, behind the fuse box.

	[image: image4.jpg]



	[image: image5.jpg]



	[image: image6.jpg]





Optional Wirings 
	Engine Immobiliz. Max 10A
	Green Wires:
	Starter: Cut the RED/BLACK wire pos.6 of the 16 pins BLACK connector linked to the central unit fixed into the steering wheel box, near the key block.

Engine: Cut the RED/GREEN wire in exit from the 7.5 A N°17 fuse linked to the back of the fuse box, fixed into the engine compartment.

	Central door locking
	Purple wire (unlock)
	Info not available

	
	Blue/Yellow wire (lock)
	

	Comfort Command

Max 100 mA
	Brown/Black Wire:
	This function it can be obtained only by the original remote control.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

