	[image: image6.png]Vy PATROLLINE 4





	FITTING 

INSTRUCTION
	Rev. 2

	
	
	

	Can Bus Diagram:  

Audi A6   2011(2018
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST 32
and next
	Remote control or keyless system version
	[image: image2.jpg]


3 [image: image3.jpg]


4
	


ELECTRIC WIRING BETWEEN ALARM AND VEHICLE
	Power
See picture 1
	Red:
	Connect to the RED/YELLOW wire of the 15A fuse N°12 into the fuse box at the left of the dashboard, on the driver side.

	
	Black:
	Connect to the original ground node, under the kickwell on the driver side.

	Enabling CAN BUS

See picture 1
	White/Green:
CAN BUS H
	Connect to ORANGE/GREEN wire pos.15 of the “C” 16 pins BROWN connector linked to the BLACK central unit, under the dashboard on the driver side.

	
	White/Brown:
CAN BUS L
	Connect to ORANGE/BROWN wire pos.14 of the “C” 16 pins BROWN connector linked to the BLACK central unit, under the dashboard on the driver side.

	Hazard
	Yellow/Green:
	Connect to the LIGHT-BLUE/WHITE wire pos. 8, in the 32-pins GREY connector located in the Black BCM unit under dashboard driver side.

	HORN command
See picture 2
	White/Red:
	Connect to BLACK/WHITE wire linked behind the relay 646 on the relay plate under the dashboard on the driver side.

	[image: image4.jpg]



	[image: image5.jpg]





Optional connections
	Central door

locking
	Violet (unlock)
Yellow/Blue (lock)
	Info not available 

	Engine Block
Max 10A
	Green Wires:
	Starter Block: Cut the RED/VIOLET wire pos.4 of the 4 pins BLACK connector, directly linke dto the brake pedal.

Engine Block: Cut the BLACK wire linked to the back of the fuse box of the 15A fuse, BLACK section, into the fuse box under the dashboard on the driver side. Use an additional Relay. 

	COMFORT Command

Max 100mA
	Brown/Black Wire:
	The closing windows can be obtained only by the original remote control.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

