|  |  |  | FITTING INSTRUCTION | Rev. 0 |
| --- | --- | --- | --- | --- |
| Can Bus Diagram: Bmw Serie 4 (G23) 2021 |  |  |  |  |
| DIP SWITCH SELECTION & SOFTWARE |  |  |  |  |
| from CARLIST 33 and next | Remote control and keyless system | 75 |  |  |

Electrical connection between alarm and vehicle

| Function | Alarm wire | Position and connection on vehicle |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power Supply Pictures 1 - 2 | Red | Connect to the RED/WHITE wire pos.33 of the 54 pins BLACK connector marked 3B linked to the central unit behind the kickwell on the passenger side. |  |  |  |
|  | Black | Connect directly to the original ground node behind the kickwell on the passenger side. |  |  |  |
| Enabling CAN BUS Pictures 1 - 3 | White/Green CAN BUS H | Connect to the YELLOW/GREEN wire pos. 50, |  | of the 54 pins BLACK connector marked 8B linked to the central unit behind the kickwell on the passenger side. |  |
|  | White/Brown CAN BUS L | Connect to the BLUE or YELLOW/BLACK wire pos. 49, |  |  |  |
| Hazard Picture 1 - 4 | Yellow/Green (Hazard) | Connect to the ORANGE wire pos.31 of the 54 puins WHITE connector marked 7B linked to the central unit behind the kickwell on the passenger side. |  |  |  |
| Horn Pictures 1 - 5 | White/Red | Connect to the RED/BROWN wire pos.3 of the 54 pins BLACK connector marked 2B linked to the central unit behind the kickwell on the passenger side. |  |  |  |
| Pic.1 Hazard + 30 Clacson GND |  |  | Pic.2 + 30 |  | Pic.3 CAN BUS |
|  |  |  | Pic. 4 HAZARD |  | Pic.5 HORN |

Additional connection

| Engine Block Max 10A | Green wires | Electric Block: cut the GREEN/WHITE wire pos. 2 of the 54 pins BLACK connector marked 3B linked to the central unit behind the kickwell on the passenger side. |
| --- | --- | --- |