|  |  |  | FITTING INSTRUCTION | Rev. 1 |
| --- | --- | --- | --- | --- |
| Can Bus: Bmw 5 series (G30) 2017 |  |  |  |  |
| DIP SWITCHES SELECTION & SOFTWARE |  |  |  |  |
| From LIST 31 and forward | Remote control & Keyless | 7 5 |  |  |

ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE

| Function | Alarm wire | Position and vehicle connection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power supplier Pic. 1 and 3 | Red | Connect to the RED wire pos 33 of the 54-pin ways BLACK connector marked “CON 3” located on the main unit placed under the kickwell passenger’s side. |  |  |  |
|  | Black | Connect directly to the original ground node placed behind the kickwell passenger’s side. |  |  |  |
| CAN BUS H Pic. 2 and 3 | White/Green | Connect to the YELLOW/RED wire pos. 50, |  | of the 54-pin ways BLACK connector marked “CON 8” located on the main unit placed under the kickwell passenger’s side. |  |
| CAN BUS L Pic. 2 and 3 | White/Brown | Connect to the YELLOW/BROWN wire pos. 49, |  |  |  |
| Blinkers Fig. 3 | Yellow/Green (Hazard) | Connect to the BROWN/BLUE wire pos.31 of the 54-pin ways WHITE connector marked “CON 7” located on the main unit placed under the kickwell passenger’s side. |  |  |  |
| HORN command | White/Red | Connect to the RED/BNROWN pos.3 of the BLACK “CON2” connector, linked to the WHITE central unit, located on the main unit placed under the kickwell passenger’s side. |  |  |  |

Optional connection:

| Central door lock | PURPLE (Locking) | Info not available yet. |
| --- | --- | --- |
|  | YELLOW/BLUE (Unlocking) |  |
| Engine kill block Max 10A | GREEN wires | Engine cut off: cut the ORANGE wire pos. 2 of the 54-pins BLACK connector marked “CON 3” located on the main unit placed under the kickwell passenger’s side. |
| COMFORT command | BROWN/BLACK | Info not available yet. |