|  |  |  | FITTING INSTRUCTION | Rev. 0 |
| --- | --- | --- | --- | --- |
| Can Bus diagram: BMW X6 (G06) 2020 |  |  |  |  |
| DIP SWITCHES SELECTION & SOFTWARE |  |  |  |  |
| From CARLIST31 and next | Remote control & Keyless | 7 5 |  |  |

ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE

| Function | Alarm wire | Position and vehicle connection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power supplier Pic. 1, 2, and 3 | Red | Connect to the RED/BLUE wire pos. 33 of the 54-pin ways BLACK connector marked “CON 3” located on the main unit placed under the kickwell passenger’s side. |  |  |  |
|  | Black | Connect directly to the original ground node placed behind the kickwell passenger’s side. |  |  |  |
| Enabling CAN BUS Pic. 1 and 4 | White/Green CAN BUS H | Connect to the YELLOW/WHITE wire pos. 48, |  | of the 54-pin ways BLACK connector marked “CON 8” located on the main unit placed under the kick wall passenger’s side. |  |
|  | White/Brown CAN BUS L | Connect to the YELLOW/BLACK wire pos. 47, |  |  |  |
| Alternative enabling CAN BUS | White/Green CAN BUS H | Connect to the YELLOW/WHITE wire pos. 36, |  | of the 54-pin ways WHITE connector marked “CON 7” located on the main unit placed under the kick wall passenger’s side. |  |
|  | White/Brown CAN BUS L | Connect to the YELLOW/BLACK wire pos. 35, |  |  |  |
| Hazard Pic. 1 and 5 | Yellow/Green | Connect to the WHITE/GREEN wire pos. 31 of the 54-pin ways WHITE connector marked “CON 7” located on the main unit placed under the kick wall passenger’s side. |  |  |  |
| HORN | White/Red | Connect to the RED/BROWN pos. 3 of the BLACK “CON2” connector, linked to the WHITE central unit, located on the main unit placed under the kick wall passenger’s side. |  |  |  |
| Horn CAN BUS Foto 1 +Power Central unit Hazard |  |  |  |  |  |
|  |  |  | Pic. 4 CAN BUS |  |  |

Optional connection:

| Engine Block Max 10A | Green wires | Starting Block: Cut the ORANGE wire pos.2 of the 54 pins BLACK connector marked “CON3”, linked to the central unit, fixed behind the kickwell on the passenger side. |
| --- | --- | --- |
|  |  | Starter motor lock (use an additional relay 30A): Cut the BLACK wire pos.1 of the 42 pins BLACK connector marked “CON2” linked to the WHITE central unit, fixed behind the kickwell on the passenger side. |