	[image: image11.png]Vy PATROLLINE 4





	FITTING 
INSRTUCTION
	Rev.1

	
	
	

	Can Bus Diagram:  

Cupra Formentor 2020(
	[image: image1.jpg]




	DIP SWITCH SELECTION & SOFTWARE
	

	From CARLIST37 and next
	Front FULL LED version
	[image: image2.jpg]


7[image: image3.jpg]


C
	

	
	No FULL LED version
	[image: image4.jpg]


7[image: image5.jpg]


9
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
Picture 1
	Red
	Connect to the RED wire 4mm² pos.66  
	The wires are located into the 73 pins BLACK connector marked “A”, linked to the central unit, under the dashboard at the left on the driver side.

	
	Black
	Connect to the BORWN 2mm² pos.63
	

	Enabling CAN BUS

Picture 1 & 2
	White/Green
CAN BUS H
	Connect to the GREEN pos.16
	

	
	White/Brown
CAN BUS L
	Connect to the ORANGE/BROWN pos.17
	

	Hazard

Selection 79
	Yellow/Green
Vehicle not equipped with FULL LED lights
	Connect to the BROWN/RED wire pos.42, of the 73 pins BLACK connector marked “C”, linked to the central unit, under the dashboard at the left on the driver side.

	Rear Blinkers
Selection 7C
Picture 4 & 5
	Fili Gialli

Vehicle equipped with FULL LED lights
	Left rear blinker: connect to the BLACK/WHITE wire pos. 60 of the 73 pins BLACK connector marked “A”, linked to the central unit, under the dashboard at the left on the driver side.
Right rear blinker: connect to the BLACK/GREEN wire pos. 27 of the 73 pins BLACK connector marked “C”, linked to the central unit, under the dashboard at the left on the driver side.

	Horn
Picture 3
	White/Red
	Attention NEGATIVE COMMAND: move the jumper on the central unit

Connect to the BROWN/RED wire pos. 43, of the 46 pins BLACK connector marked “B”, linked to the central unit, under the dashboard at the left on the driver side.

	[image: image6.jpg]



	[image: image7.jpg]



	[image: image8.jpg]




	[image: image9.jpg]



	[image: image10.jpg]





Optional Connection
	Central door locking
	VIOLET (unlock)
	Info not available at the moment

	
	YELLOW/BLUE (lock)
	

	Engine Block
Max 10A
	Green wires
	Engine Block (fuel pump petrol version): Cut the RED/BLUE wire located into the wiring loom linked to the fuel pump, into the luggage compartment, near the rear sofà.

	COMFORT Command
Max 100 mA
	Brown/Black
	Info not available at the moment


Optional connection for PATROLSAT
	Positive under key (+15)
	Orange 
	Connect to the BLACK/BLUE pos.1of the 16 pins OBD BLACK connector, under the dashboard on the driver side at the left.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

