|  |  |  | FITTING INSTRUCTION | Rev.0 |
| --- | --- | --- | --- | --- |
| Can Bus Diagram: Dacia Dokker 20122021 |  |  |  |  |
| DIP SWITCH O SOFTWARE |  |  |  |  |
| da CARLIST29 e successive | Versione con radiocomandi | 9C |  |  |

# ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE

| Function | Alarm colour wire |  | Position and vehicle connection |  |  |
| --- | --- | --- | --- | --- | --- |
| Power Supply See picure 1A/1B | Red wire |  | Connect to the RED wire pos. 12 of the 12-pin ways BROWN connector linked to the BLACK main unit under the dashboard driver’s side. |  |  |
|  | Black wire |  | Connect to the BLACK wire pos. 7 of the 12-pin ways BROWN connector connected to the BLACK main unit under the dashboard driver’s side. |  |  |
| Enabling CAN BUS | White/Green CAN BUS H |  | Connect to the wire in pos.6 of the 16-pin way OBD connector located under the dashboard driver’s side. |  |  |
|  | White/Brown CAN BUS L |  | Connect to the wire in pos.14 of the 16-pin way OBD connector located under the dashboard driver’s side. |  |  |
| Indicator light see picture 2 | Yellow wires |  | Connect to the WHITE wire pos. 5 and BROWN wire pos. 12 of the 12-pin ways WHITE connector linked to the BLACK main unit under the dashboard driver’s side. |  |  |
| Horn command | White/Red: |  | info not available yet |  |  |
| ECU view Picture 1A |  | + 30 GND Picture 1B + 30 |  | BLINKERS Picture 2 | Picture 3 |

Optional connection

| Central door looking | PURPLE unlock | NEGATIVE COMMAND ON UNIQUE WIRE. Connect the opening and closure alarm command to the GREEN wire pos. 31 of the 40-pin ways BLACK connector linked to the linked to the BLACK main unit under the dashboard driver’s side. |
| --- | --- | --- |
|  | YELLOW/BLUE lock |  |
| Engine Block Max 10A See picture 3 | Green wires: | STARTING ENGINE: cut the PINK wire pos. 3 of the 4-pin ways RED/BLACK connector connected directly to the mechanical key starting block. Check carefully the Ampere consumption while the starting command. Add a supplementary relays in order to avoid alarm mistakes. FUEL PUMP: info not available yet. |
| Comfort command Max 100 mA | Brown/Black: | Original comfort function not present. In order to rolling up the windows of the vehicle is need to add a supplementary module marked Hps44 |