	[image: image9.png]Vy PATROLLINE 4





	FITTING 
INSTRUCTION
	Rev. 0

	
	
	

	Can Bus Diagram:
Dacia Sandero III 2021(

	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	From CARLIST36 and next
	Remote controlled version
	[image: image2.jpg]


0[image: image3.jpg]


C
	


ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE

	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
Picture 1 - 2 - 3 
	Red
	Connect to the RED wire pos. 7,
	of the 16 pins GREY connector, marked DP, vertically linked to the BLACK BCM central unit fixed at the left of the steering wheel.

	
	Black
	Connect to the BLACK/WHITE wire pos. 9,
	

	Enabling CAN BUS

Foto 1 - 2 - 4
	White/Green
CAN BUS H
	Connect to the GREY/WHITE wire pos. 26,
	of the 40 pins GREY connector, marked DS1, horizontally linked to the BLACK BCM central unit fixed at the left of the steering wheel.

	
	White/Brown
CAN BUS L
	Connect to the GREEN/WHITE pos. 27,
	

	HAZARD
Foto 1 - 2 - 5
	Yellow/Green
	Connect to the BLUE/WHITE pos. 13,
	

	HORN
	White/Red
	Info not available at the moment 

	[image: image4.jpg]



	[image: image5.jpg]




	 [image: image6.emf]
	[image: image7.emf]
	[image: image8.emf]


OPTIONAL CONNECTIONS
	Central door locking
	VIOLET (unlock)
YELLOW/BLUE (lock)
	Info not available at the moment

	Engine Block
Max 10A
	GREEN wires
	Starter: Cut the LIGHT BROWN wire of the 4 pins BLACK connector, directly linked behind the START & STOP push-button in the middle of the tunnel. 


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

