	[image: image8.png]Vy PATROLLINE 4





	FITTING INSTRUCTION
	Rev.  1

	
	
	

	Can Bus:  

FORD EDGE 2016 (
	[image: image1.png]




	Attention: installed devices has to be updated at least with LIST33
	

	Installation with:

Universal cable
	Remote

or
keyless system
	[image: image2.jpg]


A[image: image3.jpg]


5
	


ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and vehicle connections

	Power

Pic. 1
	Red
	Connect to the YELLOW/RED wire 6 mm2 section, in the 1-pin BLACK connector linked to the black fuse box unit (BCM) located under dashboard driver side, at the left of the steering wheel.

	
	Black
	Connect to an original ground node. 

	CAN BUS H

Pic. 3 – 4
	White/Green

Can High
	Connect to the GREY/ORANGE wire pos. 3,
	in the BLACK 4-pins flying connector located behind the glove box passenger side, at the right.

	CAN BUS L

Pic. 3 – 4
	White/Brown

Can Low
	Connect to the PURPLE/ORANGE wire pos. 4,
	

	BLINKER 

Pic. 1 - 2
	Yellow/Green

Hazard
	Connect to the BROWN/YELLOW (or just BROWN) wire po.19, in the BLACK/GREY 52-pins connector, named “J2” linked to the black fuse box unit located under the dashboard driver side, at the left of the steering wheel.

	Horn
Pic. 6
	White/Red
	NEGATIVE COMMAND: Move the jumper on the central unit.

Link to the LIGHT BLUE wire pos.5 of the GREY connector directly linked under the steering wheel.

	[image: image9.jpg]


[image: image4.jpg]



	[image: image5.jpg]




	[image: image6.jpg]






Optional connections
	Engine Stop 

MAX 10A
	Green wires
	Start Block: cut the GREEN/RED wire of the 4 pins LIGHT BLUE brake pedal connector. 


[image: image7.jpg]



PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

