	[image: image8.png]Vy PATROLLINE 4





	FITTING INSTRUCTION
	Rev.  1

	
	
	

	Can Bus Diagram:  

FORD EXPLORER 2020 (
	[image: image1.png]




	DIP SWITCH SELECTION & SOFTWARE
	

	Updating from CARLIST33 and next
	Version with remote control & Keyless system
	[image: image2.jpg]


A[image: image3.jpg]


5
	


ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and vehicle connections

	Power

Pic. 1
	Red
	Connect to the RED wire pos.16 of the OBD connector, under the dashboard on the driver side.

	
	Black
	Connect to an original ground node behind the kick-well on the driver side.

	CAN BUS H

Pic. 1
	White/Green

Can High
	Connect to the GREY/ORANGE wire 
	in the BLACK 4-pins flying connector located behind the kick-well on the driver side.

	CAN BUS L

Pic. 1
	White/Brown

Can Low
	Connect to the PURPLE/ORANGE wire 
	

	BLINKER 

Pic. 3
	Yellow/Green

Hazard
	Connect to the BROWN/YELLOW (or just BROWN) wire po.19, in the BLACK/GREY 52-pins connector, named “J2” linked to the black fuse box unit located under the dashboard driver side, at the left of the steering wheel.

	Horn
	White/Red
	NEGATIVE COMMAND: Move the jumper on the central unit.

Link to the LIGHT BLUE wire pos.5 of the GREY connector directly linked under the steering wheel.

	[image: image9.jpg]


[image: image4.jpg]



	[image: image5.jpg]




	[image: image6.jpg]






Optional connections
	Engine Stop 

MAX 10A
	Green wires
	Start Block: cut the GREEN/RED wire of the 4 pins LIGHT BLUE brake pedal connector. 


[image: image7.jpg]



PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

