	[image: image8.jpg]



	FITTING
INSTRUCTION
	Rev.1

	
	
	

	Can Bus diagram connection:  

FORD FOCUS  2005(2011
	[image: image1.jpg]




	DIP SWITCH OR SOFTWARE SELECTION
	

	Protocol available from CARLIST30 to CARLIST36
	OEM ORIGINAL REMOTE CONTROL AND KEYLESS VERSION
	[image: image2.jpg]


1 [image: image3.jpg]


1
	


ELECTRICAL CONNECTION TO THE VEHICLE
	Function
	Alarm color wire
	Position and connection on the vehicle

	Power Supply
See picture 1
	Red:
	Connect to the 3mm ORANGE wire located into the wiring cable, behind the kickwell on the passenger side.

	
	Black:
	Connect to the original ground node of the car, behind the kickwell on the passenger side.

	Enabling CAN BUS

See picture 1
	White/Green:
	Connect to the GREY wire located into the wiring cable, behind the kickwell on the passenger side. The wire is braided with the BLUE one.

	
	White/Brown:
	Connect to the BLUE wire located into the wiring cable, behind the kickwell on the passenger side. The wire is braided with the GREY one.

	Blinkers 

See picture 3
	Yellow wires:
	Connect to the BLUE wire pos.32 and to the BLUE/RED wire pos.33 of the 46 pins BROWN connector marked “FLOOR 1”, linekd to the back of the fuse box, fixed on the passenger side.

	Horn command
See picture 2
	White/Red:
	NEGATIVE COMMAND: Move the jumper to obtain the negative alarm output

Connect to the BLACK/BLUE wire pos.6 of the 9 pins BALCK connector, fixed inside the steering wheel box.

	[image: image9.png]Vy PATROLLINE 4




[image: image4.jpg]



	[image: image5.jpg]



	
[image: image6]


Optional connection
	Engine Block
Max 10A
	Green wires:
	Petrol: Cut the GREEN/ORANGE wire directly linked to the inhertial sensor, fixed behind the glove compartment on the passenger side.
Diesel: Cut the GREEN/BLUE wire linked to the fuse N°34, linked into the fuse box of the engine compartment, on the driver side.

	Central door locking
(only for the PATROLLINE optional remote control 

HPA 974)
	VIOLET (unlocking)
YELLOW/BLUE (locking)
	Use HPA 500 module. 
Connect the VIOLET wire of the alarm to the BROWN of HPA500

Connect the YELLOW/BLUE wire of the alarm to the GREEN of HPA500

Connect the HPA500 following the central door locking diagram CH8, it’s necessary to cut the power wires of the central door locking motors.
UNLOCKING: 0,75 mmq BLACK wire pos. N°1

LOCKING: 0,75 mmq BLACK wire pos. N°2 

The wires are located into the 18 pins BLACK connector linked to the window regulator central unit, into the passenger side door. 


CH8 central door locking diagram

[image: image7.emf]Unlock

Lock

+30

M

+30

HPA500

+30


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

_1655284561.vsd
Unlock


Lock



