	[image: image7.png]Vy PATROLLINE 4





	FITTING 

INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

Hyundai Santafe’ 2012(2018
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST29
and next
	Remote control version
	[image: image2.jpg]


B[image: image3.jpg]


B
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
See picture 1
	Red:
	Connect to the RED wire 6 mm of the 4 pins WHITE connector, linked to the front of the fuse box, under the dashboard on the driver side.

	
	Black:
	Connect to the original ground node behind the kickwell on the driver side.

	Enabling CAN BUS

See picture 2
	White/Green:
CAN BUS H
	Connect to the BLUE wire pos.15 of the 24 pins WHITE connector, marked “H”, linked to the back of the fuse box, under the dashboard on the driver side.

	
	White/Brown:
CAN BUS L
	Connect to the RED wire pos. 14 of the 24 pins WHITE connector, marked “H”, linked to the back of the fuse box, under the dashboard on the driver side.

	Hazard
See picture 2
	Yellow/Green:
	Connect to the PINK wire pos.13 of the 24 pins WHITE connector, marked “H”, linked to the back of the fuse box, under the dashboard on the driver side.

	HORN 
See picture 3
	White/Red:
	Connect to the WHITE wire pos.1 of the 12 pins WHITE connector into the steering wheel box. Attention NEGATIVE COMMAND: move the jumper of the central unit.

	[image: image4.jpg]



	[image: image5.jpg]



	[image: image6.jpg]





Optional connections
	Central door locking
	VIOLET (unlock)
	Info not available at the moment

	
	YELLOW/BLUE (lock)
	

	Blocco Motore 
Max 10A
	GREEN wires
	Starter: Cut the BLACK wire pos.1 of the BLUE/BLACK connector directly linked to the key block.

	Comando COMFORT
Max 100 mA
	BROWN/BLACK
	Info not available at the moment


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

