	[image: image9.png]



	FITTING INSTRUCTION
	Rev.1

	
	
	

	Can Bus Diagram:  

Jeep Grand Cherokee 2014(2021
	[image: image1.png]




	DIP SWITCH SELECTIONS & SOFTWARE
	

	Available from CARLIST29 and next
	Remote control and Keyless Version
	[image: image2.jpg]


E[image: image3.jpg]


5
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
	Red
	To the RED wire pos. 16
	Of the OBDII 16 pins BLACK connector, fixed under the dashboard on the driver side, near to the pedals section.

	
	Black
	To the BLACK wire pos.5
	

	Enabling CAN BUS 

Till to 2018

Picture 1
	White/Green

CAN BUS H
	Connect to the GREY/WHITE pos.6 
	

	
	White/Brown

CAN BUS L
	Connect to the YELLOW wire pos.14
	

	Enabling CAN BUS 

from 2018 to 2021

Picture 4 & 5
	White/Green

CAN BUS H
	Connect to the WHITE/BROWN wire
	Of the 20 pins BROWN connector, directly linked behind the instrument panel.

	
	White/Brown

CAN BUS L
	Connect to the YELLOW wire 
	

	Hazard
Foto 3
	Yellow/Green
	Connect to the WHITE/BLUE wire of the BLACK connector directly linked behind the center console, corresponding to the HAZARD push-button.

	Horn Command
Foto 2
	White/Red
	ATTENTION: NEGATIVE command. Move the jumper on the central unit
Connect to the GREEN/BLACK wire under the steering wheel.

	[image: image10.png]


[image: image11.png]Vy PATROLLINE 4




[image: image4.jpg]


                                                         
	[image: image5.jpg]



	[image: image6.jpg]




	
[image: image7]
	
[image: image8]

	Engine Block
Max 10 A
	Green wires
	Fuel pump: Cut the BLUE/ORANGE 2.5mm connect to the 20Amp F70 fuse of the fuse box, fixed under the dashboard on the driver side, over the pedals.
Use an additional 30Amp relay.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

