	[image: image7.png]Vy PATROLLINE 4





	FITTING 

 INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram
Jeep Wrangler 2018(
	[image: image1.png]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST36
and next
	Remote control version
	[image: image2.jpg]


E[image: image3.jpg]


B
	

	
	


CONNECTION BETWEEN ALARM AND VEHICLE

	Function
	Alarm wire color
	Position and vehicle connections

	Power supply
	Red
	Connect to the RED wire 1.5mm of the free 34 pins GREY/BLACK connector fixed behind the kickwell on the passenger side. 

	
	Black
	Connect to the original ground node behind the kickwell on the passenger side.

	Enabling CAN BUS

See picture 2
	White/Green
CAN BUS H
	Connect to the BROWN wire pos.25 of the 48 pins BLACK with GREY hook connector, directly linked to the WHITE BCM central unit, fixed over the kickwell on the passenger side. 

	
	White/Brown
CAN BUS L
	Connect to the YELLOW wire pos.26 of the 48 pins BLACK with GREY hook connector, directly linked to the WHITE BCM central unit, fixed over the kickwell on the passenger side.

	Hazard
See picture 1
	Yellow/Green
	Connect to the WHITE/BLUE wire pos.21 of the 24 pins BLUE/GREY connector, directly linked to the WHITE BCM central unit, fixed over the kickwell on the passenger side.

	Horn
	White/Red 
	Info not available at the moment.

	[image: image8.jpg]


[image: image4.jpg]



	[image: image5.jpg]





Optional connection 

	Central door locking
	Violet (unlock)

Yellow/Blue (lock)
	Info not available

	Engine Block
Max 10A
	Green wires:
	Info not available

	COMFORT Command
Max 100 mA
	Brown/Black
	Info not available


[image: image6.jpg]



PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39 031 0352088  Fax: +39 031 0352089  E-mail: info@patrolline.it  Verde Italia: +39 3664966876
Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto.

L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

