	[image: image5.png]Vy PATROLLINE 4





	FITTING 
INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

Land Rover Defender   2010(2016
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST29
and next
	Analog Selection
	[image: image2.jpg]


0[image: image3.jpg]


1
	

	CAN BUS connections not available on this vehicle.

This selection has to be equipped with the additional remote controls (HPA974)
	


ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power supply
	Red:
	Connect to the VIOLET/BROWN wire pos.8 of the 12 pins GREEN connector linked to the GREEN central unit, fixed behind the speaker predisposition.

	
	Black:
	Connect to the BLACK wire pos.11 of the 12 pins GREEN connector linked to the GREEN central unit, fixed behind the speaker predisposition.

	Blinkers
	Yellow wires:
	Connect to the GREEN/WHITE wire pos.1 and GREEN/RED wire pos.6 of the 12 pins GREEN connector linked to the GREEN central unit, fixed behind the speaker predisposition.

	Positive under key (+15)
	Green/Black:
	Connect to the WHITE/GREEN wire of the 12 pins 26 pins GREY connector linked to the GREEN central unit, fixed behind the speaker predisposition. 

	Door protection
Front LEFT
	Blue:
	Connect to the GREY/WHITE wire of the 12 pins 26 pins GREY connector linked to the GREEN central unit, fixed behind the speaker predisposition.

	Door protection
Front RIGHT and rear doors
	Blue/White:
	Connect to the VIOLET/BLUE wire of the 12 pins 26 pins GREY connector linked to the GREEN central unit, fixed behind the speaker predisposition.

	Bonnet protection
	Blue/Black:
	Connect to the ORANGE/BLUE wire of the 12 pins 26 pins GREY connector linked to the GREEN central unit, fixed behind the speaker predisposition.

	Central door locking
	Violet (unlock)
Yellow/Blue (lock)
	Connect the HPA 500 module and follow the diagram CH25

Connect to the YELLOW/PINK wire of the 12 pins 26 pins GREY connector linked to the GREEN central unit, fixed behind the speaker predisposition. 

	Siren command
	White
	Connect to the WHITE wire of the HPS 95 siren.

	Engine Block
Max 10 A
	Green wires:
	Starter Block: Cut the WHITE/RED wire linked to the key block.
Engine Block: Cut the RED/GREEN wire linked to the back of the 5A fuse n°41 into the fuse box, under the dashboard on the driver side.

	COMFORT command
Max 100 mA
	Brown/Black:


	This vehicle is not equipped with the COMFORT system. To obtain the automatically windows closing is necessary to install the additional module HPS 44A. 
Front Left window: cut the RED wire pos.1 (positive of closing command) and the GREEN wire pos.5 of the 5 pins GREEN connector linked directly to the windows command push-button into the driver door.

Front right window: cut the RED wire pos.1 (positive of closing command) and the GREEN wire pos.5 of the 5 pins GREEN connector linked directly to the windows command push-button into the passenger door.


Central door locking diagram:


[image: image4.emf]POSITIVE

YELLOW/RED

Unlock

Lock

GREY

VIOLET

GREEN

YELLOW/GREY

YELLOW/BLACK

Ground

Driver door

Central unit

YELLOW/PINK

YELLOW/PINK

BROWN

HPA 500

VIOLET

ALARM

YELLOW/BLUE

ALARM

YELLOW/BLUE

Do not connect


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

_1702369939.vsd

