	[image: image7.png]Vy PATROLLINE 4





	FITTING
INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

Lexus GS  2012(2020
	[image: image1.jpg]Hun3D






	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST29
and next
	Remote control version
	[image: image2.jpg]


2[image: image3.jpg]


4
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
	Red:
	Connect to the RED wire pos.16 of the 16 pins OBD connector, fixed under the dashboard on the driver side at the left.

	
	Black:
	Connect to the BROWN wire pos.5 of the 16 pins OBD connector, fixed under the dashboard on the driver side at the left.

	Enabling CAN BUS

See picture 2 & 3
	White/Green: 
CAN BUS H
	Connect to the VIOLET wire pos.14 of the middle WHITE connector linked to the BCM central unit under the dashboard on the driver side.

	
	White/Brown: 
CAN BUS L
	Connect to the WHITE wire pos.13 of the middle WHITE connector linked to the BCM central unit under the dashboard on the driver side.

	HORN command
See picture 1
	White/Red:
	NEGATIVE COMMAND: move the jumper on the dashboard
Connect to the YELLOW wire of the 14 pins BLACK connector, linked into the steering wheel box. 

	[image: image4.jpg]



	[image: image5.png]



	[image: image6.png]





Optional connection
	Central door locking
	Violet (unlock): 
	Connect to the VIOLET wire of the 12 pins WHITE connector, fixed behind the kickwell on driver side.

	
	Yellow/Blue (lock):
	Connect to the GREEN wire of the 12 pins WHITE connector, fixed behind the kickwell on driver side.

	Engine Block
Max 10A
	Green wires:
	Starter: Cut the RED wire of the 4 pins GREEN connector, linked to the brake micro-switch.
Engine: Cut the RED wire located into the wiring loom, under the dashboard on driver side.

	COMFORT Command
Max 100 mA
	Brown/Black:
	Info not available yet


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

