	[image: image10.png]Vy PATROLLINE 4





	FITTING

INSTRUCTIONS
	Rev. 4

	
	
	

	Can Bus Diagram:  

Mercedes C Class (w205) 2014(2021
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST29

and next
	2014 -> 2019

Remote or/and Keyless go
	[image: image2.jpg]


5[image: image3.jpg]


6
	

	from CARLIST33

and next
	2019 ->

Remote or/and Keyless go
	[image: image4.jpg]


5[image: image5.jpg]


8
	


ELECTRIC WIRING BETWEEN ALARM AND VEHICLE

	Function
	Alarm wire
	Position and vehicle connections

	Power supply 

See picture 1
	Red
	Connect to the RED wire of the 2 pins WHITE connector linked in front of the BMC main unit located to the left side on the driver side.

	
	Black
	Connect to the BROWN wire of the 2 pins WHITE connector linked in front of the BMC main unit located to the left side on the driver side.

	Enabling CAN BUS

See picture 1 - 3
	White/Green
CAN BUS H
	Connect to BROWN/RED wire of the 52 pins YELLOW connector marked RBA1, connected in front of the BCM main unit to the left side on the driver side. Alternatively connect to the BROWN/RED wire located in the wiring loom under the door sill passenger side (see picture 3). The wire is always twisted with the BROWN one.

	
	White/Brown
CAN BUS L
	Connect to the BROWN wire of the 52 pins YELLOW connector marked RBA1, connected in front of the BCM main unit to the left side on the driver side. Alternatively connect to the BROWN wire located in the wiring loom under the door sill passenger side (see picture 3). The wire is always twisted with the BROWN/RED one.

	HORN
See picture 2
	White/Red
	Positive command. Connect to the YELLOW wire of the 12 pins PURPLE connector linked on the front of the fuse box, into the engine compartment on the driver side.

	[image: image6.jpg]



	[image: image7.jpg]




	[image: image8.jpg]





Optional Connections: 
	Engine Kill 
Max 10A 
	Green wires
	Engine kill (petrol and diesel): Cut the PINK/RED wire pos. 2 of the 10 pins BLUE connector linked at the rear of the fuse box, into the engine compartment on the driver side.

	Comfort
	Brown/Black
	The no IK vehicles version isn’t equipped with COMFORT function.


Rev 2 aggiunta selezione 58

Rev 3 tolta selezione 58 – aggiornati collegamenti CAN

Rev 4 aggiunta doppia selezione 56 e 58 (SM – 19/08/19)
[image: image9.jpg]



PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY Tel: +39.031.0352088 Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

