	[image: image1.jpg]y PATROLLINE





	
Rev.1
FITTING

INSTRUCTION

	Universal 

Can Bus 

DIP SWITCH SELECTION

[image: image2.jpg]


 9

[image: image3.jpg]


 B


	[image: image4.jpg]





Mercedes Citan 2012 (2021
ELECTRIC WIRING BETWEEN ALARM AND VEHICLE

	Power supply
	Red Wire:
	Connect to RED wire pos.16 of the 16 pins OBD BLACK connector, fixed on the central tunnel, near the 12V plug.

	
	Black Wire:
	Connect to BLACK wire pos.5 of the 16 pins OBD BLACK connector, fixed on the central tunnel, near the 12V plug.

	Can Bus L

See picture 1
	White/Brown Wire:
	Connect to BROWN wire pos.14 of the 16 pins OBD BLACK connector, fixed on the central tunnel, near the 12V plug.

	Can Bus H

See picture 2
	White/Green Wire:
	Connect to ORANGE wire pos.6 of the 16 pins OBD BLACK connector, fixed on the central tunnel, near the 12V plug.

	Blinkers
See picture 2
	Yellow Wires:
	Connect to the WHITE/BLACK wire of the wiring loom, located under the kickwell to the rear, of the passenger side and to the GREY/BLUE wire of the wiring loom, located under the kickwell to the rear, on the driver side.

Or connect to GREY/BLUE and WHITE/BLACK wires located in the pos. 2 and 3 of the Black/Red connector named  "P203" on the BCM unit at the left of the steering wheel under the dashboard driver side.

	Horn Command

	White/Red Wire:
	Connect to PURPLE (or BROWN in some vehicles) wire of the wiring loom in exit from the steering wheel box.

	[image: image5.jpg]



	[image: image6.jpg]





Optional Wirings for Mercedes Citan from 2012
	Central door locking
	Purple wire (unlock)
	Connect to the YELLOW/BLACK wire of the 8 pins BLACK connector linked to the lock/unlock push-button, located on the central dashboard.

	
	Blue/Yellow wire (lock)
	

	Engine Immobiliz.

10A
	Green Wires:
	Cut the YELLOW wire of the N°22 5A fuse linked to the fuse box located, at the left, under the dashboard on the driver side.

	Comfort command
	Brown/Black wire:
	The automatic closing of the windows is possible only by the original remote control.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

