	[image: image6.png]Vy PATROLLINE 4





	FITTING 
INSTRUCTION
	Rev. 0

	
	
	

	Can Bus Diagram:  

MG ZS EV 2018(
	[image: image1.jpg]




	DIP SWITCH SELECTION & SOFTWARE
	

	Available from CARLIST37 and next
	Keyless & Remote-control version 
	[image: image2.jpg]


0[image: image3.jpg]


B
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
	Red
	Connect to the wire pos.16 
	Of the 16 pins OBD BLACK connector

	
	Black
	Connect to the wire pos.4
	

	Enabling CAN BUS

Picture 1
	White/Green
CAN BUS H
	Connect to the YELLOW/BLACK wire pos.3
	Of the GREY connector linked to the BCM central unit, fixed behind the glove box, on the passenger side.

	
	White/Brown
CAN BUS L
	Connect to the GREEN/BROWN wire pos.2
	

	Hazard
Picture 2
	Yellow/Green
Hazard
	Connect to the PINK/BLACK wire pos.33 of the LIGH BLUE connector, BLACK section, linked to the BCM central unit, fixed behind the glove box, on the passenger side.

	Horn
	White/Red
	Info not available

	[image: image7.jpg]


[image: image4.png]



	[image: image5.jpg]





PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

