	[image: image8.jpg]



	FITTING
INSTRUCTION
	Rev. 4

	
	
	

	Can Bus Diagram:  

Mercedes Classe C (w206) 2021(
	[image: image1.png]




	DIP SWITCH SELECTION & SOFTWARE
	

	From CARLIST38 
and next
	Remote control and Keyless version
	[image: image2.jpg]


5[image: image3.jpg]


D
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function 
	Alarm wire 
	Position and connection on vehicle

	Power Supply

See picture 1
	Red
	Connect to the RED 2mm 
	Of the 63 pins LIGHT BLUE connector marked “RB2”, linked to the CAS central unit, fixed under the dashboard, at the left on high.

	
	Black
	Connect to the BROWN 2mm
	

	Enabling CAN BUS

See picture 1
	White/Green
CAN BUS H
	Connect to the VIOLET/WHITE pos.11
	Of the 32 pins WHITE connector marked “RB2”, linked to the CAS central unit, fixed under the dashboard, at the left on high.

	
	White/Brown
CAN BUS L
	Connect to the GREY wire pos.10
	

	Blinkers

See picture 3
	Yellow wires
	Attention: for this vehicle is possible to turn ON only the rear blinkers.

Connect to the VIOLET/BLACK pos.60 of the 63 pins WHITE connector “RB1” and to the GREEN/WHITE pos.17 of the 63 pins LIGHT BLUE connector “RB2”. These connectors are linked to the GREY central unit, fixed into the rear compartment at the right.

	HORN

See picture 1
	White/Red
	Connect to the YELLOW/GREEN wire pos.8 of the 31 pins VIOLET connector marked “MR2”, linked to the CAS central unit, fixed under the dashboard, at the left on high.

Attention let use an additional relay of at least 20A to command the horn. 

See the drawing
	
[image: image4.emf]85 –Connect to the WHITE/RED wire of the alarm

86 –Connect to the ground -31

30 –Connect to the positive fixed +30

87 –Connect to the horn command of the vehicle

87A



	[image: image9.png]Vy PATROLLINE 4




[image: image10.jpg]


[image: image5.jpg]



	
[image: image6]

	
	
[image: image7]


Optional connection
	Engine Block
Max 10A
	Green wires:
	Engine Block: Cut the PINK wire linked to the to the 5A fuse on the back of the fuse box, fixed into the engine compartment on the driver side. See picture 2

	COMFORT command
Max 100mA
	Brown/Black:
	This function is available by the original remote control.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

_1716727868.vsd
87A


85 – Connect to the WHITE/RED wire of the alarm


86 – Connect to the ground -31


30 – Connect to the positive fixed +30


87 – Connect to the horn command of the vehicle



