	[image: image7.emf]
	FITTING 
INSTRUCTIONS
	Rev. 0

	
	
	

	Can Bus Diagram:  

Mercedes GLA (H247)2020(
	[image: image1.jpg]-

Mercedes GLA H247





	DIP SWITCH SELECTION & SOFTWARE
	

	from CARLIST35
and next
	Remote Control and Keyless version
	[image: image2.jpg]


5[image: image3.jpg]


3
	


[image: image8.png]Vy PATROLLINE 4




 ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
See picture 1
	Red
	Connect to the RED wire big section, located under the moquette, under the glove box on the passenger side.

	
	Black
	Conenct to the original ground node, under the moquette, under the glove box on the passenger side.

	Enabling CAN BUS

See picture 1 & 2
	Green/White

CAN BUS H
	Connect to the BROWN/RED wire of the 10 pins WHITE connector, under the moquette, under the glove box on the passenger side. The wire is braided with the BROWN one.

	
	Brown/White

CAN BUS L
	Connect to the BROWN wire of the 10 pins WHITE connector, under the moquette, under the glove box on the passenger side. The wire is braided with the BROWN/RED one.

	HORN command
	White/Red:
	Info not available at the moment

	
[image: image4]
	[image: image5.jpg]



	[image: image6.jpg]





Optional connection
	Central door locking
	Violet (unlock)
	It’s not possible to command the central door locking

	
	Yellow/Blue (lock)
	

	Engine Block
Max 10A
Foto 3
	Green wires:
	Engine Block: Cut the PINK/RED wire in exit from the relay marked 032, fixed into the fuse box of the engine compartment.

	COMFORT command
	Brown wire:
	This function i obtained only by the original remote control.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

