	[image: image6.png]Vy PATROLLINE 4





	FITTING

INSTRUCTIONS
	Rev. 0

	
	
	

	Can Bus Diagram:
Mercedes “GLE” (W167) 2019(
	[image: image1.png]




	DIP SWITCHES SELECTION OR SOFTWARE
	

	From Car List 33
And next
	Remote control and

Keyless version
	[image: image2.jpg]


5[image: image3.jpg]


8
	


ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and vehicle connection

	Power supply
	Red
	Connect to the RED wire 2.5mm into the wiring under the upholstery of the footrest on the passenger side.

	
	Black
	Connect to original ground node under the upholstery of the footrest on the passenger side.

	Enabling CAN BUS
	White/Green 
CAN BUS H
	Connect to BROWN/RED wire inserted in one of the CAN BUS leads under the upholstery of the footrest on the passenger side. The wire is braided with the BROWN one.

	
	White/Brown 
CAN BUS L
	Connect to BROWN wire inserted in one of the CAN BUS leads under the upholstery of the footrest on the passenger side. The wire is braided with the BROWN/RED one.

	Horn
	White/Red
	Info not available.

	[image: image4.jpg]





Optional connections:
	Central door 
locking
	Violet (unlock)
Yellow/Blue (lock)
	Info not available at the moment

	Engine Immobilizer 

Max 10A 
	Green Wires
	Starter Kill: Cut the RED/GREEN wire 2.5mm of the 4 pins GREY connector linked to the back of the fuse box into the engine compartment, driver side.

	COMFORT command

Max 100 mA
	Brown/Black 
	Info not available at the moment


[image: image5.jpg]



PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because isn’t synchronized to changes, modifications or supplementary devices installed in the vehicles from manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

