	[image: image11.png]Vy PATROLLINE 4





	FITTING

INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

Mercedes Vito (w639)   2004(2014
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	Horn command for all CARLIST
	Remote control & Keyless version
	[image: image2.jpg]


5[image: image3.jpg]


1
	

	Siren command from CARLIST30 and next
	
	[image: image4.jpg]


5[image: image5.jpg]


2
	

	Siren command till to CARLIST29
	
	[image: image6.jpg]


5[image: image7.jpg]


3
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power supply
See picture 1
	Red:
	Connect to the RED wire of the 2 pins BLACK connector fixed under the glove box on the passenger side.

	
	Black:
	Connecto to the BROWN wire directly linked to the original groud node behind the kickwell on the passenger side.

	Enabling CAN BUS
See picture 2
	White/Green:
CAN BUS H
	Connect to the BROWN/RED wire of the BLACK connector into the multi-plug fixed under the glove box on the passenger side. The wire is braided with the BROWN one.

	
	White/Brown:
CAN BUS L
	Connect to the BROWN wire of the BLACK connector into the multi-plug fixed under the glove box on the passenger side. The wire is braided with the BROWN/RED one.

	[image: image8.jpg]



	[image: image9.jpg]





Optional connections
	Engine Block 
Max 10A
	Green wires:
	Diesel: Cut the BLACK/YELLOW wire of the 10A n°40 fuse into the VIOLET fuse holder, fixed to the side of the fuse box into the engine compartment, passenger side.

	COMFORT command
Max 100mA
	Brown/Black:
	the original windows closing is obtained only by the original remote control

	Central door locking
	Violet (unlock)
Yellow/Blue (lock)
	Connect both wires to the GREEN/BLACK wire, directly linked to the original lock/unlock push-button.


[image: image10.jpg]



PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

