	[image: image9.png]Vy PATROLLINE 4





	FITTING 

INSTRUCTION
	Rev. 2

	
	
	

	Can Bus Diagram:  

Nissan Micra  2010(2017
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST29
and next
	Remote control version
	[image: image2.jpg]


4[image: image3.jpg]


7
	

	
	Keyless version
	[image: image4.jpg]


3[image: image5.jpg]


1
	


ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
See picture 1
	Red:
	Connect to the GREEN wire pos. B6 of the 15 pins BLACK connector linked to the BCM central unit, under the dashboard at the left of the steering wheel.

	
	Black:
	Connect to the BLACK wire pos.5 of the 16 pins OBD II connector, fixed under the dashboard at the left.

	Enabling CAN BUS

See picture 1
	White/Green:
	Connect to the BLUE wire pos.6 of the 16 pins OBD II connector, fixed under the dashboard at the left.

	
	White/Brown:
	Connect to the PINK wire pos.14 of the 16 pins OBD II connector, fixed under the dashboard at the left.

	Blinkers
See picture 1
	Yellow wires:
	Connect to the VIOLET wire pos.A5 and WHITE wire pos.A6 of the 15 pins BLACK connector linked to the BCM central unit, under the dashboard at the left of the steering wheel.

	HORN command
See picture 2
	White/Red:
	NEGATIVE COMMAND: move the jumper on the central unit. Connect to the RED wire of the 8 pins GREY connector linked into the steering wheel box. 

	[image: image6.jpg]



	[image: image7.jpg]



	[image: image8.jpg]





  Optional connections 

	Engine Block

Max 10A
	Green wires:
	Starter:

Cut the LIGHT BLUE wire pos.3 of the 4 pins WHITE or BLACK connector over the pedals.
Engine:

Cut the ORANGE wire of the “ELEC IG ECM” 10A fuse of the fuse box under the dashboard at the left.

	COMFORT command
Max 100mA
	Brown/Black:
	this vehicle is not equipped with the COMFORT command. To obtain the automatically windows closing, it’s necessary to use the additional HPS 44A module.

Front left window: RED wire (positive command) of the 16 pins WHITE conenctor, directly linked to the windows command push-button inside the driver door. Front right window: GREY wire (positive command) located into the wiring loom in exit from the driver door. 


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

