	[image: image8.png]Vy PATROLLINE 4





	FITTING 
INSTRUCTION
	Rev. 1

	
	
	

	Can Bus Diagram:  

Nissan Note  2013(2020
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST29
and next
	Remote control version
	[image: image2.jpg]


4 [image: image3.jpg]


7
	

	
	Keyless Version
	[image: image4.jpg]


3 [image: image5.jpg]


1
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
	Red:
	Connect to the GREEN wire pos. B6 of the 15 pins WHITE connector, linked to the BCM central unit, under the dashboard on driver side at the left.

	
	Black:
	Connect to the original ground node under the kickwell on driver side.

	Enabling CAN BUS
	White/Green:
CAN BUS H
	Connect to the BLUE wire pos.6 
	Of the 16 pins OBD connector, fixed under the dashboard on the driver side.

	
	White/Brown:
CAN BUS L
	Connect to the PINK wire pos.14 
	

	Blinkers
Vedi foto 2
	Yellow wires:
	Connect to the GREEN pos.A5 and WHITE pos. A6 wires of the 15 pins WHITE connector, linked to the BCM central unit, under the dashboard on driver side at the left, or to the GREEN and WHITE wires of the 24 pins WHITE connector behind the kickwell on driver side.

	HORN command
Vedi foto 1
	White/Red:
	NEGATIVE command: move the jumper on central unit.

Connect to the RED wire of the 6 pins YELLOW connector under the wheel at the right, into the steering wheel box. 

	[image: image6.jpg]



	[image: image7.jpg]





Optional connections
	Central door locking
	Violet (unlock)
	Connect to the BROWN wire pos.A6 
	Of the 15 pins BLACK connector, linked to the BCM central unit, under the dashboard on driver side at the left.

	
	Yellow/Blue (lock)
	Connect to the GREY wire pos.A5 
	

	Engine Block
Max 10A
	Green wires:
	Engine Block: Cut the ORANGE wire of the 10A fuse, marked “ELECTRIC PARTS (IGN) ECW/DC DCIGN”.

	COMFORT command
Max 100mA
	Brown/Black:
	Vehicle not equipped with COMFORT command. To obtain the automatically windows closing it’s necessary to use the additional HPS 44A module. 

Front left window: RED wire (positive command) of the 16 pins WHITE connector directly linked to the windows command push-button, into the driver side door; Front right window: WHITE wire (positive command); Rear left window: BROWN wire (positive command); Rear right window: GREY wire (positive command) These wires are located behind the kickwell on driver side.


Rev 1: aggiunta alternative collegamento frecce (SM  19/08/2019)
PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

