	[image: image4.png]Vy PATROLLINE 4





	FITTING 

INSTRUCTION
	Rev.0

	
	
	

	Diagram Connection:  

Opel Vivaro 2007(2015
	[image: image1.jpg]




	DIP SWITC SELECTION AND SOFTWARE
	

	from CARLIST29
and next
	
	[image: image2.jpg]


0 [image: image3.jpg]


1
	

	ATTENTION: This selection allows to arm and disarm the alarm only with the additional remote control HPA 974.
The Original remote control cannot command the alarm.
	


  ELECTRICAL CONNECTION ON THE VEHICLE
	Function
	Alarm wire color
	Position and connection on the vehicle

	Power Supply
	Red
	Connect to the RED wire 6mm of the 4 pins BLACK connector, into the steering wheel box.

	
	Black
	Connect to the original ground node, under the kickwell on the driver side.

	Positive under key
	Green/Black
	Connect to the YELLOW wire of the 4 pins BLACK connector, into the steering wheel box.

	Blinkers command
	Yellow Wires
	Connect to the VIOLET wire pos. A4 and BROWN wire pos. A3 of the 36 pins BLACK/LIGHT BLUE connector, linked near the hood open lever. 

	Doors protection
(Separated lines)
	Blue
Separate the lines with 

the diodes
	Connect to the BROWN wire (Ant. left) pos. A8 of the 36 pins BLACK/LIGHT BLUE connector, linked near the hood open lever and to the BROWN/RED wire (Ant. Right) into the wiring loom behind the passenger side central pillar cover, near the door buttons.

	Trunk Protection

(Separated lines)
	Blue/White

Separate the lines with 

the diodes
	Connect to the VIOLET wire (tailgate) pos. A9 of the 36 pins BLACK/LIGHT BLUE connector, linked near the hood open lever and to the GREEN/RED wire (side door) into the wiring loom behind the passenger side central pillar cover, near the door buttons.

	Central door 

locking
	Violet (unlock)

Yellow/Blue (lock)
	Do the connection diagram CH1/N. 

Connect the lock wire to the ORANGE wire pos. A3 and the unlock wire to the WHITE wire pos. B1 of the 6 pins BLACK connector, linked to the internal central door locking push-button, fixed on the central tunnel. 

	Engine Block
Max 10A
	Green Wires
	Engine Block: Cut the RED wire of the “UCH INK” 10A fuse, linked to the of the fuse box, fixed on the side of the dashboard on the driver side.
Starter Block: Connect to the WHITE wire of the 4 pins BLACK connector, into the steering wheel box.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

