	[image: image5.png]Vy PATROLLINE 4





	Fitting instructions for 

CAN BUS device
	Rev. 0

	Opel Vivaro 2005 ( 2014
	[image: image1.jpg]




	DIP SWITCH A: F - DIP SWITCH B: 0
	

	
[image: image2.emf]F

B

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A

0

 HPS 845 R
	
[image: image3.emf]0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A B

F 0


HPS 869
	

	Available from CARLIST29 & next
	

	It’s necessary to memorize al least one additional remote control HPA 974 to make the alarm functions operational
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply

Picture 1
	Red
	Connect to the wire pos.16
	Of the 16 pins OBD connector, under the steering wheel at the left.

	
	Black
	Connect to the wire pos.4
	

	CAN BUS connection
Picture 3
	White/Green

Can BUS H
	Connect to the wire pos.6
	

	
	White/Brown

Can BUS L
	Connect to the wire pos.14
	

	Blinkers

Picture 2
	Yellow wires
Blinkers
	Connect to the VIOLET pos. A4 and HAZELNUT pos. A3 wires in the 36 pins free BLACK/LIGHT BLUE connector, fixed near to the hood unlocking mechanical lever.

	Doors protection
	Blue

Use the diodes to separate the lines
	Front Left: connect to the HAZELNUT wire pos. A8
Trunk: Connect to the VIOLET wire pos. A9
	In the 36 pins free BLACK/LIGHT BLUE connector, fixed near to the hood unlocking mechanical lever

	
	
	Front right: connect to the HAZELNUT/RED wire
Rear lateral right: connect to the GREEN/RED wire
	In the wiring loom behind the kickwell on the passenger side, near to the doors push-button.

	Hood protection
	Blue/Black
	Install a mechanical push-button on the side member in the engine compartment

	Central door locking connection
	Violet (unlocking)
	Connect to the WHITE wire pos. B1
	Of the 6 pins BLACK connector linked to the internal lock/unlock push-button, fixed on the middle of the center console.

	
	Yellow/Blue (locking)
	Connect to the ORANGE wire pos. A3
	

	HORN command
	White/red
	Info not available at the moment
	
[image: image4.emf]

	Setting 

push-button 
	Grey/Black

Black
	Connect the GREY/WIRE wire of push-button to the BRWON wire of alarm. 
Connect the BLACK wire to the ground.


      Optional connections
	Engine Block
Max 10A
	Green wires
	Engine block: cut the RED wire linked to the back of the fuse box, fixed on the side of the dashboard, behind the 10A fuse marked “UCH INJ”.


External devices connection

	HPS 94C siren
	Connect the WHITE wire of HPS 845R central unit to the BROWN wire of siren. 

	HPS 98 siren
	No wires connections, this is a WIRELESS device.

The memorization sequence:
· Driver door opened, key OFF and alarm disarmed
· Push 2 times the setting push-button, the LED on the dashboard starts to flash for 30 seconds
· Put 10 times the BROWN wire of the siren to the ground
· The siren gives some bips to confirm the memorization ok

	Patrolsat 4G
	Connect the WHITE wire of HPS 845R central unit to the VIOLET wire of PatrolSat 4G SOS push-button.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

_1734954059.vsd
F


A


0


1


2


3


4


5


6


7


8


9


A


B


C


D


B


0


E


F


1


2


0


3


4


5


6


7


8


9


A


B


C


D


E


F



_1734954075.vsd
B


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


A


F


0



_1728281815.vsd

