	[image: image13.png]Vy PATROLLINE 4





	FITTING 
INSTRUCTION
	Rev. 0

	
	
	

	Can Diagram:  

Opel Vivaro 2019 (
	[image: image1.jpg]




	DIP SWITCH SELECTION & SOFTWARE
 Updating from CARLISTA31 and next 
	

	Horn Command – via CAN BUS
	[image: image2.jpg]


C [image: image3.jpg]


2
	

	Siren command
	[image: image4.jpg]


C [image: image5.jpg]


3
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function 
	Alarm wire
	Position and connection on vehicle

	Power Supply
Picture 1
	Red
	Connect to the BLACK wire 4 mm2 pos.2 of the BLACK/RED connector marked “PB2 BLACK” linked to the BCM central unit under the dashboard on the driver side.

	
	Black
	Connect to the original ground node under the dashboard on the driver side.

	Enabling CAN BUS
Picture 3
	White/Green
Can H
	Connect to the GREY wire. pos. 3,
	into the 4 pins GREY connector under the steering wheel into the box.

	
	White/Brown
Can L
	Connect to the GREEN wire pos. 1,
	

	Hazard
Picture 2 & 4
	Yellow/Green
	Connect to the VIOLET (or RED) wire pos.7, of the 60 pins BLUE connector marked “EPB BLUE” linked to the BCM central unit under the dashboard on the driver side.

	Inhibition
Picture 1 & 5
	Blue
	ATTENTION: mandatory connection to obtain tha alarm disarms
Connect to the GREEN pos. 11 (or BROWN pos. 9) of the 60 pins BROWN connector marked “EH1 BROWN” linked to the BCM central unit under the dashboard on the driver side.
ATTENTION: connect a 10 KΩ resistance between this wire and the RED one.
Without this connection the alarm could disarms when you lock again the central door locking.

	Front Door
(Ceiling light)

foto 6 e 7
	Blue/Red
	Connect to the PINK wire pos. 35 of the BLACK conenctor marked “PAV BLACK” nella centralina sotto il cruscotto lato guida.

	Trunk
foto 6 e 7
	Green/Black
	Collegarsi al filo ROSA di sezione più piccola in pos. 34 del connettore Nero “PAV BLACK” linked to the BCM central unit under the dashboard on the driver side.

	HORN command and siren
	White/Red
	Sel. C2 – Do not connect. Function obtained via CAN BUS.

Sel. C3 – Connect to the wire command of the siren.

	Engine Block
Max 10A
	Green wires
	Starter Block: 

Cut the VIOLET wire pos.22 of the 60 pins BLUE connector marked “EPB BLUE” linked to the BCM central unit under the dashboard on the driver side


NOTE:
· When the alarm is armed and you open the trunk by the original remote control the system trigger. Unlock the central door locking before to open the trunk.
	[image: image6.jpg]




	[image: image7.jpg]





	[image: image8.jpg]




	[image: image9.jpg]





	
[image: image10.jpg]



	[image: image11.jpg]



	[image: image12.jpg]





PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

