	[image: image13.png]Vy PATROLLINE 4





	FITTING

INSTRUCTION
	Rev. 3

	
	
	

	Can Bus:  

Peugeot 308 2013(2021
	[image: image1.jpg]





	DIP SWITCHES OR SOFTWARE SELECTION
 Update to LIST31 or forward
	

	With 

Remote Control
 only
	Horn driving
	[image: image2.jpg]


C [image: image3.jpg]


2
	

	
	Siren driving
	[image: image4.jpg]


C [image: image5.jpg]


3
	

	With 
Remote Control 
and with
Keyless Entry System
	Horn driving
	[image: image6.jpg]


2 [image: image7.jpg]


E
	

	
	Siren driving
	[image: image8.jpg]


2 [image: image9.jpg]


D
	


 CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire colour
	Position and vehicle connections

	Power supplier

Pic. 2
	Red
	Connect to RED wire in pos. 2 of the Black connector named PB2, linked to the BSI central unit, fixed under the dashboard on the driver side.

	
	Black
	Connect to and ground node.

	CAN BUS L

Pic.  1
	White/Brown

(Can Low)
	Connect to the GREEN wire pos. 1,
	in the 4 pins Gray connector under the wheel box

	CAN BUS H
Pic. 1
	White/Green

(Can High)
	Connect to the GRAY wire pos. 3,
	

	Blinker

(Hazard button)
Pic. 2
	Yellow/Green
	Connect to the PURPLE wire pos. 7, in the 60 pins Blue connector named EPB BLUE, linked to the BSI central unit, fixed under the dashboard on the driver side.

	Lock state
Pic. 2
	Blue
or Blue/Red for 840U

Attention: connection only for selection C2/C3

(vehicles with remote only)
	Negative wire when vehicle is unlocked:
Connect to the LIGHT BROWN wire pos. 13, or to the wire in pos. 15, in the 60 pins Brown connector named “EH1 BROWN", linked to the BSI central unit, fixed under the dashboard on the driver side.

or:
Connect to the GREY wire pos. 33 in the 60 pins Blue connector named "EPB", linked to the BSI central unit, fixed under the dashboard on the driver side.

Attention: if connection is missing, the device fails disarming

	Doors

(dome light)
	Green/Black

Attention: connection only for selection C2/C3

(vehicles with remote only)
	Positive input
Connect to the double WHITE wire coming from the connector in the left roof pillar.

	Engine Immobilizer

Pic. 2
	Green wires
	Starter 

Cut the BLUE wire pos.22,
	in the 60 pins Blue connector named "EPB", linked to the BSI central unit, fixed under the dashboard on the driver side.

	
	
	Fuel Pump 

Cut the BLUE wire pos.32,
	

	Horn

or
Siren
	Red/White
	Sel. C2/2E -  Not connect - function available by CAN BUS (no beeps).
Sel. C3/2D - Connect to the proper Siren trigger wire.

	Doors locking/unlocking

(optional connections if you add additional remote control only)
	Yellow/Blue

(locking)
	Connect both wires to the YELLOW wire pos. 33, in the 60 pins Blue connector named "EPB", linked to the BSI central unit, fixed under the dashboard on the driver side.

	
	Purple  

  (unlocking) 
	


  See Pag.2

	
[image: image10.jpg]


 

[image: image11.jpg]



	[image: image12.jpg]Cometore EH1
EH1 connector
Comnecteur EH1
Conector EH1

Comnetiore PB2
PB2 connector
Connecieur PB2
Conector PB2








PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

