	 [image: image7.png]Vy PATROLLINE 4





	FITTING 

INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

PEUGEOT BOXER  2011( 2014
	[image: image1.png]




	DIP SWITCH SELECTION OR SOFTWARE 
	

	from CARLIST29
and next
	Remote controlled version
	[image: image2.jpg]


1[image: image3.jpg]


7
	


CONNECTION BETWEEN ALARM AND VEHICLE
	Power supply
Picture 1
	Red 
	Connect to the RED/GREEN wire pos. 16 of the 16 pins OBD connector located under the dashboard driver’s side.

	
	Black 
	Connect to the BLACK/VIOLET wire pos. 5 of the 16 pins OBD connector located under the dashboard driver’s side.

	Enabling CAN BUS

Picture 1
	White/Green 
CAN BUS H
	Connect to the BLUE wire pos. 1 of the 16 pins OBD connector located under the dashboard driver’s side.

	
	White/Brown 
CAN BUS L
	Connect to the WHITE wire pos. 9 of the 16 pins OBD connector located under the dashboard driver’s side.

	Hazard
Picture 2
	Yellow/Green 
	Connect to the PURPLE/ORANGE wire pos.55 of the 60 pins GREY connector connected on the back of the fuse box driver’s side.

	Horn Command
Picture 3
	White/Red 
	Negative command. Connect to the Green/Black wire located in the BLACK connector under the steering wheel.

	[image: image4.jpg]



	[image: image5.jpg]



	[image: image6.jpg]





Optional wiring
	Engine Block 

10A
	Green Wires
	Starter: Cut the BROWN/GREY wire, directly linked to the key block.

Engine: Cut the WHITE/BLACK wire, directly linked to the key block.

	Comfort command
Max 100mA
	Brown/Black
	Info not available


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

