	[image: image7.png]Vy PATROLLINE 4





	FITTING 

INSTRUCTION
	Rev.0

	
	
	

	Can Bus fitting instruction:  

Renault Fluence 2010(2017

	[image: image1.jpg]




	DIP SWITCH SELECTION
	

	Update from LIST29 and upper
	
	[image: image2.jpg]


9[image: image3.jpg]


A
	


ELECTRIC WIRING BETWEEN ALARM AND VEHICLE

	Function
	Alarm wire colour
	Position and vehicle connection 

	Power supply
	Red Wire:
	Connect to the RED wire located in exit to the bracket of relay support.

	
	Black Wire:
	Connect to the GROUND original node behind the kickwell driver side.

	CAN BUS

See picture 1
	White/Brown:
	Connect to the GREEN wire pos. 1 of the 40-pin ways BLACK connector linked to the BLACK main unit located horizontally under the dashboard driver’s side.

	
	White/Green:
	Connect to the GREY wire pos. 3 of the 40-pin ways BLACK connector linked to the BLACK main unit located horizontally under the dashboard driver’s side.

	Blinkers
See picture 2
	Yellow wires:
	Connect to the light BLUE wire pos. 1 and PINK pos. 5 of the 12-pin ways connector marked “PB1 BLACK” connected to the BLACK main unit fixed horizontally under the dashboard driver’s side.

	Horn Command
See picture 3
	White/Red:
	NEGATIVE COMMAND. Please check carefully the JUMPER polarity of the alarm.
Connect to the White wire pos 10 of the 12-pin ways GREY connector located inside of the steering wheels black box 

	[image: image4.jpg]



	[image: image5.jpg][
b

|2




	[image: image6.jpg]





Optional connection 
	Engine Block 
Max 10A
	Green wires:
	To cut the YELLOW wire GIALLO pos. 4 of the 12-pin ways GREY connector connected in front of the fuse box located in the engine compartment.

	COMFORT command 
Max 100mA
	Brown wire:
	Info not available


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

