	[image: image7.png]Vy PATROLLINE 4





	FITTING INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

Renault Trafic  2021(
	[image: image1.png]




	DIP SWITCH SELECTION & SOFTWARE
	

	From CARLIST36 and next
	Remote control version & keyless
	[image: image2.jpg]


0 [image: image3.jpg]


C
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power supply
	Red:
	Connect to the RED wire 0.75mm 
	Of the 16 pins GREY connector, marked “DP”, linked to the BCM central unit, fixed at the right of the fuse box, under the dashboard on the driver side.

	
	Black:
	Connect to the BLACK wire 0.75mm 
	

	Enabling CAN BUS

See picture 2
	White/Green:
CAN BUS H
	Connect to the GREEN wire pos 26 
	Of the 40 pins GREY connector marked “DS1” linked to the BLACK central unit, fixed under the dashboard on the driver side at the left of the steering column.

	
	White/Brown:
CAN BUS L
	Connect to the BROWN wire pos. 27 
	

	Hazard
	Yellow/Green:
	Connect to the BLUE wire pos.13 
	

	HORN command
See picture 3
	White/Red:
	NEGATIVE COMMAND: Move the jumper on the central unit.

Connect to the WHITE wire pos.10 of the 12 pins GREY connector, linked to the central unit under the steering wheel. 

	[image: image4.png]



	[image: image5.jpg]



	[image: image6.jpg]





Optional connection
	Central door locking 
	Violet Unlock
	Connect both of alarm wire to the GREEN/WHITE wire pos.12 of the GREEN connector, horizontally linked to the BLACK central unit vertically fixed at the left of the steering wheel.

	
	Yellow/Blue Lock
	

	Engine Block
Max 10A
	Green wires:
	Starter Block: Use an added 30A Relay. Cut the WHITE wire pos.6 of the 6 pins GREY connector marked “MM”, linked to the front of the central unit into the engine compartment, driver side.
Engine Block: Cut the YELLOW wire pos.1 of the 24 pins GREY connector marked “CT1”, linked to the front of the central unit into the engine compartment, driver side. 

	COMFORT command
Max 100 mA
	Brown/Black:
	The automatically lock of the windows is obtained only by the original remote control.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

