	[image: image7.png]



	FITTING 
INSTRUCTION
	Rev.0

	
	
	

	Can Bus diagram:  

Suzuki S-Cross 2013(2021
	[image: image1.jpg]




	DIP SWITCH SELECTION & SOFTWARE
	

	Updating from CARLIST29 and next 
	Remote control version
	[image: image2.jpg]


A[image: image3.jpg]


1
	


ELECTRIC WIRING BETWEEN ALARM AND VEHICLE

	Power Supply
See picture 1
	Red Wire:
	Connect to the 6mm WHITE wire of the 2 pins WHITE connector, linked to the front of the central unit, fixed under the dashboard at the left. 

	
	Black Wire:
	Connect to the original ground node, near to the OBD connector, under the dashboard on the driver side.

	CAN BUS connection
Until 2021

See picture 1
	White/Green Wire: 
CAN BUS H
	Connect to the RED wire pos.6. This wire is braided with the WHITE one pos.14.
	of the 16 pins BLACK OBD connector, fixed under the dachboard on the driver side at the left.

	
	White/Brown Wire:
CAN BUS L
	Connect to the WHITE wire pos.14.This wire is braided with the RED one pos.6.
	

	Alternative CAN BUS from 2021

See picture 2
	White/Green Wire: 
CAN BUS H
	Connect to the RED. This wire is braided with the WHITE one.
	Of the 24 pins BLACK connector linked to the BLACK central unit, fixed at the left of the steering column, under the dashboard on the driver side.

	
	White/Brown Wire:
CAN BUS L
	Connect to the WHITE wire. This wire is braided with the RED.
	

	Horn Command

See picture 3
	White/Red Wire:
	Connect to the LIGHT BLUE wire pos.4 of the 12 pins BLACK connector, linked into the steering wheel box. 
Be careful with the NEGATIVE CONTROL: for the working, move the inside JUMPER of the central unit.

	[image: image8.png]Vy PATROLLINE 4




[image: image4.jpg]



	
[image: image5]
	[image: image6.jpg]





Optional connections
	Central door locking
	Purple wire (unlock)
	Connect to the GREY wire of the wiring loom under the kickwell on the driver side.

	
	Blue/Yellow wire (lock)
	Connect to the GREEN wire of the wiring loom under the kickwell on the driver side.

	Engine Immobiliz. Max 10A
	Green Wires:
	Starter Block: Cut the GREEN wire of the 2 pins WHITE connector, directly linked to the clutch pedal. Only for the vehicles equipped with “KEYLESS GO” system and manual gear.

Engine Block: Cut the WHITE wire pos.5 of the 30 pins WHITE/BLACK connector, linked to the top of the central unit, fixed at the left, under the dashboard on the driver side.

	Modules Command
	Brown Wire:
	This vehicle is not equipped with the COMFORT function.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

