	[image: image8.png]Vy PATROLLINE 4





	Fitting instruction for CAN BUS device installation
	Rev. 0

	
	
	

	Suzuki Vitara 2015(
	[image: image1.png]




	DIP SWITCH A: A - DIP SWITCH B: 1
	

	
[image: image2.emf]A

B

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A

1

HPS 845 R
	
[image: image3.emf]0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A B

A 1


HPS 869
	

	Available from CARLIST29 and next
	

	Hybrid Version with keyless & Remote control
	


Electrical connection between alarm and vehicle
	Power Supply
	Red
	Connect to the YELLOW wire pos.16 of the OBD connector, under the dashboard at the left.

	
	Black
	Connect to the original ground node, behind the kickwell, under the unlocking bonnet lever.

	CAN BUS connection
	White/Green 
CAN BUS H
	Connect to the RED wire pos. 15 
	of the 32-pins GREY connector, located in the BLACK BCM unit under the dashboard on the driver side.

	
	White/Brown
CAN BUS L
	Connect to the WHITE wire pos. 31 
	

	Alternative CAN BUS connection

See picture 2
	White/Green 
CAN BUS H
	Connect to the RED wire pos.12
	of the 18 pins BLACK connector fixed behind the kickwell, under the unlocking bonnet lever.

	
	White/Brown
CAN BUS L
	Connect to the WHITE wire pos.18 
	

	Blinkers
	Yellow wires
	Do not connect, function obtained by the CAN BUS command

	Horn Command

See picture 3
	White/Red
	Attention NEGATIVE command. Move the jumper on the central unit. Connect to the BLUE wire pos.4 of the 12 pins BLACK connector, linked to the central unit under the steering wheel.
	
[image: image4.emf]

	Setting push-button 
	Grey/Black

Black
	Connect the GREY/BLACK wire of the push-button to the BROWN wire of the alarm wiring.

Connect the BLACK wire of the push-button to the ground.

	[image: image5.jpg]



	[image: image6.jpg]



	[image: image7.jpg]





Optional connections
	Central door locking
	Violet (unlock)
	Connect to the GREY wire of the wiring loom under the kickwell on the driver side.

	
	Blue/Yellow (lock)
	Connect to the GREEN wire of the wiring loom under the kickwell on the driver side.

	Engine Block

Max 10A
	Green Wires:
	Starter Block: Cut the GREEN wire of the 2 pins WHITE connector, directly linked to the clutch pedal. Only for the vehicles equipped with “KEYLESS GO” system and manual gear.

Engine Block: Cut the WHITE wire pos.5 of the 30 pins WHITE/BLACK connector, linked to the top of the central unit, fixed at the left, under the dashboard on the driver side.


External devices connections

	HPS 94C siren
	Connect the WHITE wire of the HPS 845R alarm system to the BROWN of the siren 

	HPS 98 siren
	No wire connection, wireless siren.

The storage steps are listed below:

· Driver door opened, key OFF and alarm disarmed
· Push 2 times the setting push-button, the alarm LED starts to flash quickly for 30 seconds
· Put the BROWN wire of the siren to the ground for 10 times and wait for the special bips of memorization confirming

	Patrolsat 4G
	Connect the WHITE wire of the HPS 845R alarm system to the VIOLET wire of the SOS PatrolSat 4G


Rev. 1: cambiata connessione CAN (S.M. 25/03/19) 

Rev.2: aggiunta alternativa CAN BUS modello 2022 (S.D.C. 18/01/2023)
PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because it does not synchronized to changes, modifications or installations of car manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

_1735549042.vsd
A


A


0


1


2


3


4


5


6


7


8


9


A


B


C


D


B


0


E


F


1


2


1


3


4


5


6


7


8


9


A


B


C


D


E


F



_1735549083.vsd
B


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


A


A


1



_1724737310.vsd

