
	[image: image7.png]Vy PATROLLINE 4





	FITTING

INSTRUCTIONS
	Rev. 1

	
	
	

	Can Bus Diagram:  

Toyota C-HR      2017(
	[image: image1.jpg]




	DIP SWITCHES SELECTION OR SOFTWARE
	

	From CARLIST32
and forward
	With remote control or

with keyless system
	[image: image2.jpg]


2 [image: image3.jpg]


4
	


ELECTRICAL CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position in the vehicle

	Power supply

See pic. 1
	Red
	Connect to the RED wire in the 63-pins GREY connector located under kick wall driver side.

	
	Black
	Connect to the original ground node located under kick wall driver side.

	CAN BUS

See pic. 3
	White/Green

(CAN H)
	Connect to the PURPLE wire pos. 32
	in the 40-pins BLACK connector behind the dashboard panel.

	
	White/Brown

(CAN L)
	Connect to the LIGHT BLUE wire pos. 31
	

	Alternative:

CAN BUS
	White/Green 

(CAN H)
	Connect to the GREEN wire pos. 14
	in the 30-pins connector locate on the body control unit under dashboard diver side.

	
	White/Brown

(CAN L)
	Connect to the BLUE wire pos. 13
	

	Blinkers

See pic. 2
	Yellow/Green

(Hazard)
	Do not connect, blinker’s function is obtained by Can Bus Command, if not use connection to the hazard wire located to the WHITE wire in the 4-pins GREY connector behind the hazard pushbutton in the middle of the console.

	Horn
	White/Red
	Connect to GREEN wire pos. 8 of the 12-pins BLACK connector located inside the steering wheel box.
Command has to be NEGATIVE: set the alarm JUMPER according.

	[image: image4.jpg]



	[image: image5.jpg]



	[image: image6.jpg]





Optional Connections:
	Engine Block 
Max 10A
	Green wires
	Starter Block: Cut the BLACK wire pos.13 of the 27 pins WHITE connector linked to the BLACK central unit, fixed vertically, behind the glove compartment on the passenger side.

	COMFORT Command
	Brown/Black
	Info not available.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because isn’t synchronized to changes, modifications or supplementary devices installed in the vehicles from manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

