	[image: image7.png]Vy PATROLLINE 4





	Fitting instructions for CAN BUS devices instalaltion
	Rev. 0

	
	
	

	Toyota Corolla Cross 2022(
	[image: image1.jpg]




	DIP SWITCH A: 6 - DIP SWITCH B: B
	

	
[image: image2.emf]6

B

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A

B

 HPS 845 R
	
[image: image3.emf]0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A B

6 B


HPS 869
	

	Available from CARLIST39 and next
	

	Keyless and remote control version Petrol Engine
	


Electrical connection between alarm and vehicle
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
	Red
	Connect to the RED wire pos.16 of the OBD 16 pins connector linked under the dashboard on the driver side, at the left.

	
	Black
	Connect to the BLACK/WHITE wire pos.4 of the OBD 16 pins connector linked under the dashboard on the driver side, at the left or connect to the original ground node behind the kickwell on the driver side.

	Enabling CAN BUS

See pictures
	White/Green 
CAN BUS H
	Connect to the BLACK wire
	Of the WHITE connector directly linked to the instrument panel.

	
	White/Brown
CAN BUS L
	Connect to the WHITE wire
	

	Blinkers
	Yellow wires
	Do not connect. Function obtained by the CAN BUS command.

	Horn
	White/Red
	NEGATIVE COMMAND: Move the jumper on the central unit.

Connect to the GREEN wire pos.8 of the 12 pins BLACK connector, linked to the central unit under the steering wheel, into the box.
	
[image: image4.emf]

	Setting push-button 
	Grey/Black

Black
	Connect the GREY/BLACK wire of the push-button to the BROWN wire of the alarm wiring.

Connect the BLACK wire of the push-button to the ground.

	[image: image5.jpg]



	[image: image6.jpg]





Optional Connections
	Central door locking
	Violet (Unlock)

Yellow/Blue (lock)
	Info not available at the moment

	Engine Block

Max 10A
	Green wires:
	Info not available at the moment


External devices connections

	HPS 94C siren
	Connect the WHITE wire of the HPS 845R alarm system to the BROWN of the siren 

	HPS 98 siren
	No wire connection, wireless siren.

The storage steps are listed below:

· Driver door opened, key OFF and alarm disarmed
· Push 2 times the setting push-button, the alarm LED starts to flash quickly for 30 seconds
· Put the BROWN wire of the siren to the ground for 10 times and wait for the special bips of memorization confirming

	Patrolsat 4G
	Connect the WHITE wire of the HPS 845R alarm system to the VIOLET wire of the SOS PatrolSat 4G


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

_1728974982.vsd
6


A


0


1


2


3


4


5


6


7


8


9


A


B


C


D


B


0


E


F


1


2


B


3


4


5


6


7


8


9


A


B


C


D


E


F



_1728974996.vsd
B


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


A


6


B



_1724737310.vsd

