	[image: image9.png]Vy PATROLLINE 4





	Fitting instructions for CAN BUS devices installation
	Rev. 0

	
	
	

	Toyota Corolla Cross 2022(
	[image: image1.png]




	DIP SWITCH A: 6 - DIP SWITCH B: B
	

	
[image: image2.emf]6

B

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A

B

 HPS 845 R
	
[image: image3.emf]0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

0

1

2

3

4

5

6

7

8

9

A

B

C

D

E

F

A B

6 B


HPS 869
	

	Available from CARLIST39 and next
	

	Keyless and remote control version Petrol Engine
	


ELECTRICAL CONNECTION FROM ALARM AND VEHICLE
	Function
	Alarm wire colour
	Position and connection on the vehicle

	Power Supply
Picture 1
	Red:
	Connect to the BLACK wire pos.16 of the 16 pins OBD WHITE connector, fixed under the dashboard on the driver side at the left.

	
	Black:
	Connect to the WHITE/BLACK wire pos.4 of the 16 pins OBD WHITE connector, fixed under the dashboard on the driver side at the left.

	Enabling CAN BUS

Picture 2
	White/Green:
CAN BUS H
	Connect the BROWN wire of the 38 pins WHITE connector, linked behind the instrument panel.

The wire is braided with the WHITE one.

	
	White/Brown:
CAN BUS L
	Connect the WHITE wire of the 38 pins WHITE connector, linked behind the instrument panel.

The wire is braided with the BROWN one.

	Blinkers
	Yellow wires:
	DO NOT CONNECT, function obtained through the CAN BUS command.

	HORN command

Picture 3
	White/Red:
	NEGATIVE COMMAND: move the jumper on the alarm central unit. 
Connect to the BLACK wire pos.8 of the 12 pins BLACK connector linked into the steering box under the steering wheel.
	
[image: image4.emf]

	Setting push-button 
	Grey/Black

Black
	Connect the GREY/BLACK wire of the push-button to the BROWN wire of the alarm wiring.

Connect the BLACK wire of the push-button to the ground.

	 [image: image5.emf]
	[image: image6.emf]
	[image: image7.jpg]





Optional connections
	Central door locking
	Violet (unlock)
Blue/Yellow (lock)
	Info not available at the moment

	Engine Block 10A
	Green wires:
	Starter Block: Cut the BROWN wire on the 4 pins BLACK connector directly linked to the START & STOP push button, located on the central tunnel.

Petrol and Diesel: cut the BLACK o GREY wire 1mm, located into the wiring loom under the kickwell on the driver side.

Hybrid Version: Cut the BLACK/WHITE wire on the brake pedal (this interruption prevents the use of the gearbox) 

	Modules Command

Max 100 mA
	Brown/Black:
	The COMFORT function is obtained through the original remote control.

	Positive under key +15
	Connect to the BLACK wire pos.12 of the 45 pins WHITE connector, linked to the front of the central unit, fixed under the dashboard at the left of the steering wheel. 
	[image: image8.emf]


External devices connections

	HPS 94C siren
	Connect the WHITE wire of the HPS 845R alarm system to the BROWN of the siren 

	HPS 98 siren
	No wire connection, wireless siren.

The storage steps are listed below:

· Driver door opened, key OFF and alarm disarmed
· Push 2 times the setting push-button, the alarm LED starts to flash quickly for 30 seconds
· Put the BROWN wire of the siren to the ground for 10 times and wait for the special bips of memorization confirming

	Patrolsat 4G
	Connect the WHITE wire of the HPS 845R alarm system to the VIOLET wire of the SOS PatrolSat 4G


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

_1728974982.vsd
6


A


0


1


2


3


4


5


6


7


8


9


A


B


C


D


B


0


E


F


1


2


B


3


4


5


6


7


8


9


A


B


C


D


E


F



_1728974996.vsd
B


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


0


1


2


3


4


5


6


7


8


9


A


B


C


D


E


F


A


6


B



_1724737310.vsd

