	[image: image8.png]Vy PATROLLINE 4





	FITTING
INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

Volkswagen Polo Sedan  2010(2015
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	From CARLIST29 and next
	Remote control version
	[image: image2.jpg]


3[image: image3.jpg]


3
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire
	Position and connection on vehicle

	Power Supply
See picture 1
	Red:
	Connect to the RED/YELLOW wire pos.73 
	Of the 73 pins BLACK connector linked to the BCM central unit, fixed over the pedals.

	
	Black:
	Connect to the BROWN wire pos.70 
	

	Enabling CAN BUS

See picture 3
	White/Green:
CAN BUS H
	Connect to the ORANGE/BLACK wire pos.19 
	Of the 73 pins WHITE connector linked to the BCM central unit, fixed over the pedals.

	
	White/Brown:
CAN BUS L
	Connect to the ORANGE/BROWN wire pos.18 
	

	Blinkers

See picture 2
	Yellow wires:
	Connect to the BLACK/GREEN wire pos.32 
	Of the 73 pins BLACK connector linked to the BCM central unit, fixed over the pedals.

	HORN command
See picture 1
	White/Red:
	Connect to the BLACK/YELLOW wire pos.72 
	

	[image: image4.jpg]



	[image: image5.jpg]



	[image: image6.jpg]





Optional connection
	Engine Block
Max 10A
	Green wires:
	Cut the LIGHT BLUE/WHITE wire linked to the fuse n°31 of the fuse box at the left of the steering wheel.

	COMFORT command
Max 100mA
	Brown/Black:
	the automatically windows closing is obtained only by the original remote control.


[image: image7.jpg]



PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

