	[image: image7.png]Vy PATROLLINE 4





	FITTING
INSTRUCTION
	Rev.0

	
	
	

	Can Bus Diagram:  

Volkswagen Multivan 2010(2015
	[image: image1.jpg]




	DIP SWITCHES SELECTION & SOFTWARE
	

	from CARLIST29
and next
	Remote control version
	[image: image2.jpg]


3[image: image3.jpg]


0
	


ELECTRICAL CONNECTION BETWEEN ALARM AND VEHICLE

	Function 
	Alarm wire
	Position and connection on vehicle

	Power Supply
	Red:
	Connect to the RED/WHITE wire pos.16 of the 17 pins BLACK connector, fixed on the connector holder bracket, under the dashboard on the driver side. 

	
	Black:
	Connect to the original ground node fixed at the left of the friction pedal.

	Enabling CAN BUS

See picture 1
	White/Green: 

CAN BUS H
	Connect to the ORANGE/GREEN pos.20 of the 73 pins WHITE connector, marked “B”, linked to the central unit, under the dashboard on the driver side. 

	
	White/Brown:

CAN BUS L
	Connect to the ORANGE/BROWN pos.21 of the 73 pins WHITE connector, marked “B”, linked to the central unit, under the dashboard on the driver side.

	Hazard

See picture 3
	Green/Yellow:
	Connect to the WHITE/GREEN pos.3 of the 6 pins BLACK connector, directly lined to the hazard push button.

	HORN command

See picture 2
	White/Red:
	Connect to the BLACK/YELLOW pos.72 of the 73 pins BLACK connector, amrked “A”, linked to the central unit, under the dashboard on the driver side.

	[image: image4.jpg]



	[image: image5.jpg]



	[image: image6.jpg]





Optional connections
	Central door locking
	Violet (unlock)

Yellow/Blue (lock)
	Connect the unlock and lock wire of the alarm to the YELLOW wire pos.8 of the 17 pins BLACK connector, fixed on the connector holder bracket, under the dashboard on the driver side.

	Engine Block

Max 10A
	Green wires:
	Cut the BLACK wire directly linked to the key block.

	COMFORT command

Max 100 mA
	Brown/Black:
	Connect to the YELLOW/BLU wire pos.6 of the 17 pins BLACK connector, fixed on the connector holder bracket, under the dashboard on the driver side.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

