	[image: image6.png]Vy PATROLLINE 4





	FITTING

INSTRUCTION
	Rev. 0

	
	
	

	Can Bus Diagram:  

Volvo V60 2019 (
	[image: image1.png]




	DIP SWITCHES OR SOFTWARE SELECTION
	

	From CARLIST33
and next
	Remote Controls
and
Keyless Version
	[image: image2.jpg]


9[image: image3.jpg]


7
	


CONNECTIONS BETWEEN ALARM AND VEHICLE
	Function
	Alarm wire colour
	Position and vehicle connections

	Power supply
	Red
	Connect to the wire in the position 16 in the diagnostic OBD connector.

	
	Black
	Connect to the original ground node under the kickwell on the driver side.

	Enabling CAN BUS
Pic. 1
	White/Green

CAN BUS H
	Connect to the VIOLET/WHITE wire
	Into the wiring loom under the kickwell on the driver side.

	
	White/Brown

CAN BUS L 
	Connect to the VIOLET/GREEN wire
	

	Hazard
Pic. 2
	Yellow/Green
	Connect to the BROWN/GREY wire pos. 6 located in the 10-pins GREEN connector behind the keyboard for multimedia control where is located the hazard pushbutton.

	Horn
	White/Red
	Info not available.

	[image: image4.jpg]¢





	[image: image5.jpg]





  Optional connections:
	Engine Block
Max 10A
	Green wires
	Info not available.

	Comfort Command

Max 100mA
	Brown/Black
	Info not available.


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089

E-mail: info@patrolline.it Web: www.patrolline.it The information given on this sheet is for guidance only and not binding because isn’t synchronized to changes, modifications or supplementary devices installed in the vehicles from manufacturers. The company is not liable for any damage to the vehicle resulting from incorrect installation.

