	[image: image9.png]Vy PATROLLINE 4





	FITTING

 INSTRUCTION
	Rev.0

	
	
	

	Can Bus data seet:  

NISSAN NV200 2014 (
(comfort e premium version)


	[image: image1.jpg]




	DIP SWITCH SELECTION AND SOFTWARE
	

	From CARLIST29
And after

	With remote control 
	[image: image2.jpg]


4 [image: image3.jpg]


7
	

	
	With intelligent key 
	[image: image4.jpg]


3 [image: image5.jpg]


1
	

	
	


CONNECTION BETWEEN ALARM AND VEHICLE
	Function
	Alarm colour wire
	Posizione e connessione su vettura

	Power supplier
	Red wire:
	Connect to the YELLOW wire of the 12-pin ways WHITE connector linked to the main unit placed under the dashboard passenger side.

	
	Black wire:
	Connect to the original ground node present behind the kickwell passenger side.

	CAN BUS H
See pictures 1
	White/Green wire:
	Connect to the BLUE wire pos.22 of the 40-pin ways WHITE connector linked to the main unit placed under the dashboard passenger side.

	CAN BUS L
See pictures 1
	White/Brown wire:
	Connect to the ROSE wire pos.21 of the 40-pin ways WHITE connector linked to the main unit placed under the dashboard passenger side.

	Blinker
See pictures 2
	Yellow wires:
	Connect to the GREEN and WHIRE wire of the 18-pin ways BLACK connector linked to the main unit placed under the dashboard passenger side.

	HORN command
See pictures 3
	White/Red wire
	Connect to the RED wire of the 6-pin ways GREY connector placed inside the steering wheel black box. NEGATIVE command: check carefully the polarity output siren 

	[image: image6.emf]
	[image: image7.jpg]



	[image: image8.jpg]





Optional connection
	Central door locking 
	PURPLE wire (opening)
	info not available yet

	
	YELLOW/BLUE (closing)
	info not available yet

	Engine block 
Max 10A
	GREEN wire:
	Starting Engine: cut the WHITE wire of the 6-pin ways WHITE connector linked directly to the mechanical key block

	COMFORT command
	BROWN wire:
	info not available yet


PATROLLINE GROUP Società Cooperativa - Via C. Cantù 15/C 22031 Albavilla (CO) ITALY  Tel: +39.031.0352088  Fax: +39.031.0352089  Assistenza: +39 3664966876

E-mail: info@patrolline.it      Web: www.patrolline.it      Le indicazioni riportate su questa scheda sono da considerarsi indicative e non vincolanti poiché non sincronizzate a modifiche, variazioni o allestimenti delle case auto. L’azienda non risponde di eventuali danni al veicolo derivanti da una non corretta installazione.

